UNIVERSITY OF WAH

Department: Artificial Intelligence

Name of Assignment: Project

Name: Muhammad Hamza Sajjad

Subject: Computer Networks

# Network Implementation Report

# Topology/Confriguration

# Total Network Components Summary:

## Total Devices: 34 devices = 19 Routers (Cisco 2811 with WIC-2T modules) + 7 Switches (Cisco 2960-24TT) + 3 Access Points (AP-PT) + 3 Servers (1 DHCP, 1 Mail, 1 Web) + Various End Devices (PCs, Laptops, Smartphones, Tablets)

## 1. VLSM Calculations

### Public IP Addressing (Router-to-Router)

The network 192.168.1.148/30 has 2 hosts.

Your subnets need 44 hosts.

| Name | Hosts Needed | Hosts Available | Unused hosts | Network Address | Slash | Mask | Usable Range | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 2 | 2 | 0 | 192.168.1.148 | /30 | 255.255.255.252 | 192.168.1.149 - 192.168.1.150 | 192.168.1.151 |
| 1 | 2 | 2 | 0 | 192.168.1.152 | /30 | 255.255.255.252 | 192.168.1.153 - 192.168.1.154 | 192.168.1.155 |
| 2 | 2 | 2 | 0 | 192.168.1.156 | /30 | 255.255.255.252 | 192.168.1.157 - 192.168.1.158 | 192.168.1.159 |
| 3 | 2 | 2 | 0 | 192.168.1.160 | /30 | 255.255.255.252 | 192.168.1.161 - 192.168.1.162 | 192.168.1.163 |
| 4 | 2 | 2 | 0 | 192.168.1.164 | /30 | 255.255.255.252 | 192.168.1.165 - 192.168.1.166 | 192.168.1.167 |
| 5 | 2 | 2 | 0 | 192.168.1.168 | /30 | 255.255.255.252 | 192.168.1.169 - 192.168.1.170 | 192.168.1.171 |
| 6 | 2 | 2 | 0 | 192.168.1.172 | /30 | 255.255.255.252 | 192.168.1.173 - 192.168.1.174 | 192.168.1.175 |
| 7 | 2 | 2 | 0 | 192.168.1.176 | /30 | 255.255.255.252 | 192.168.1.177 - 192.168.1.178 | 192.168.1.179 |
| 8 | 2 | 2 | 0 | 192.168.1.180 | /30 | 255.255.255.252 | 192.168.1.181 - 192.168.1.182 | 192.168.1.183 |
| 9 | 2 | 2 | 0 | 192.168.1.184 | /30 | 255.255.255.252 | 192.168.1.185 - 192.168.1.186 | 192.168.1.187 |
| 10 | 2 | 2 | 0 | 192.168.1.188 | /30 | 255.255.255.252 | 192.168.1.189 - 192.168.1.190 | 192.168.1.191 |
| 11 | 2 | 2 | 0 | 192.168.1.192 | /30 | 255.255.255.252 | 192.168.1.193 - 192.168.1.194 | 192.168.1.195 |
| 12 | 2 | 2 | 0 | 192.168.1.196 | /30 | 255.255.255.252 | 192.168.1.197 - 192.168.1.198 | 192.168.1.199 |
| 13 | 2 | 2 | 0 | 192.168.1.200 | /30 | 255.255.255.252 | 192.168.1.201 - 192.168.1.202 | 192.168.1.203 |
| 14 | 2 | 2 | 0 | 192.168.1.204 | /30 | 255.255.255.252 | 192.168.1.205 - 192.168.1.206 | 192.168.1.207 |
| 15 | 2 | 2 | 0 | 192.168.1.208 | /30 | 255.255.255.252 | 192.168.1.209 - 192.168.1.210 | 192.168.1.211 |
| 16 | 2 | 2 | 0 | 192.168.1.212 | /30 | 255.255.255.252 | 192.168.1.213 - 192.168.1.214 | 192.168.1.215 |
| 17 | 2 | 2 | 0 | 192.168.1.216 | /30 | 255.255.255.252 | 192.168.1.217 - 192.168.1.218 | 192.168.1.219 |
| 18 | 2 | 2 | 0 | 192.168.1.220 | /30 | 255.255.255.252 | 192.168.1.221 - 192.168.1.222 | 192.168.1.223 |
| 19 | 2 | 2 | 0 | 192.168.1.224 | /30 | 255.255.255.252 | 192.168.1.225 - 192.168.1.226 | 192.168.1.227 |
| 20 | 2 | 2 | 0 | 192.168.1.228 | /30 | 255.255.255.252 | 192.168.1.229 - 192.168.1.230 | 192.168.1.231 |
| 21 | 2 | 2 | 0 | 192.168.1.232 | /30 | 255.255.255.252 | 192.168.1.233 - 192.168.1.234 | 192.168.1.235 |
| 22 | 2 | 2 | 0 | 192.168.1.236 | /30 | 255.255.255.252 | 192.168.1.237 - 192.168.1.238 | 192.168.1.239 |

Private IP Addressing (Networks)

The network 198.32.0.0/24 has 254 hosts.

Your subnets need 558896 hosts.

| Name | Hosts Needed | Hosts Available | Unused hosts | Network Address | Slash | Mask | Usable Range | Broadcast |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Network E | 92345 | 131070 | 38725 | 198.32.0.0 | /15 | 255.254.0.0 | 198.32.0.1 - 198.33.255.254 | 198.33.255.255 |
| Network D | 81234 | 131070 | 49836 | 198.34.0.0 | /15 | 255.254.0.0 | 198.34.0.1 - 198.35.255.254 | 198.35.255.255 |
| Network C | 70123 | 131070 | 60947 | 198.36.0.0 | /15 | 255.254.0.0 | 198.36.0.1 - 198.37.255.254 | 198.37.255.255 |
| Network B | 69012 | 131070 | 62058 | 198.38.0.0 | /15 | 255.254.0.0 | 198.38.0.1 - 198.39.255.254 | 198.39.255.255 |
| Network A | 58901 | 65534 | 6633 | 198.40.0.0 | /16 | 255.255.0.0 | 198.40.0.1 - 198.40.255.254 | 198.40.255.255 |
| Network K | 58901 | 65534 | 6633 | 198.41.0.0 | /16 | 255.255.0.0 | 198.41.0.1 - 198.41.255.254 | 198.41.255.255 |
| Network J | 47890 | 65534 | 17644 | 198.42.0.0 | /16 | 255.255.0.0 | 198.42.0.1 - 198.42.255.254 | 198.42.255.255 |
| Network I | 36789 | 65534 | 28745 | 198.43.0.0 | /16 | 255.255.0.0 | 198.43.0.1 - 198.43.255.254 | 198.43.255.255 |
| Network H | 25678 | 32766 | 7088 | 198.44.0.0 | /17 | 255.255.128.0 | 198.44.0.1 - 198.44.127.254 | 198.44.127.255 |
| Network G | 14567 | 16382 | 1815 | 198.44.128.0 | /18 | 255.255.192.0 | 198.44.128.1 - 198.44.191.254 | 198.44.191.255 |
| Network F | 3456 | 4094 | 638 | 198.44.192.0 | /20 | 255.255.240.0 | 198.44.192.1 - 198.44.207.254 | 198.44.207.255 |

Network Blocks and Connections (with Redistribution Roles)

RIP

- Router 0:

  - Connected to Switch 0 (Network A).

    - Switch 0 connects to:

      - Laptop 0

      - Laptop 1

  - Connected to Router 1 and Router 2.

- Router 1:

  - Connected to Switch 1 (Network B).

    - Switch 1 connects to:

      - Laptop 2

      - Laptop 3

  - Connected to Switch 2 (Network C).

    - Switch 2 connects to:

      - PC 2

      - PC 3

      - Mail Server

  - Connected to Router 0, Router 3.

- Router 2:

  - Connected to Router 0 and Router 3.

- Router 3:

  - Connected to Router 1 , Router 2 and Router 4.

- Router 4:
(Redistribution Router between RIP and OSPF Area 1)

  - Connected to Router 3, and Router 5.

OSPF Area 1

- Router 5:

  - Connected to Router 4 and Router 6.

- Router 6:

  - Connected to Router 5, 7 and Router 8.

- Router 7:

  - Connected to Router 6, Router 9, and Switch 3 (Network D).

    - Switch 3 connects to:

      - PC 4

      - CopyLaptop 1

- Router 8 (NAT) :

  - Connected to Access Point 2 (Network E).

    - Access Point 2 connects to:

      - Tablet PC 4

      - Tablet PC 5

      - Smartphone 2

  - Connected to Router 6,9 and Router 10.

- Router 9:

  - Connected to Router 7,8 ,10 and Switch 4 (Network F).

    - Switch 4 connects to:

      - CopyPC 4

      - CopyCopyLaptop 1

- Router 10:
(Redistribution Router between OSPF Area 1 and EIGRP)

  - Connected to:

    - Router 8

    - Router 9

    - Router 11

EIGRP

- Router 11:

  - Connected to Router 10, 12,13 and Router 18.

- Router 12:

  - Connected to Access Point 1 (Network G).

    - Access Point 1 connects to:

      - Smartphone 3

  - Connected to:

    - Router 11

- Router 13:

  - Connected to:

    - Router 11

    - Switch 5 (Network H).

      - Switch 5 connects to:

        - Server 1

    - Switch 6 (Network I).

      - Switch 6 connects to:

        - PC5

        - PC6

- Router 18:
(Redistribution Router between EIGRP and OSPF Area 2)

  - Connected to:

    - Router 11,17

OSPF Area 2

- Router 14:

  - Connected to Switch 7 (Network K).

    - Switch 7 connects to:

      - PC 7

      - PC 8

      - DHCP Server

  - Connected to:

    - Router 15

    - NAT-Router 16

- Router 15:

  - Connected to:

    - Router 14

    - Nat-Router 16

    - Router 17

- Router 16:

  - Acts as NAT Router.

  - Connected to:

    - Router 14

    - Router 15

    - Router 17

  - Connected to Access Point 0 (Network J).

    - Access Point 2 connects to:

      - Smartphone 5

      - Smartphone 4

- Router 17:

  - Connected to:

    - Router 15

    - Router 16

    - Router 18

Key Devices and Servers

- Mail Server:

  - Connected to Switch 2 (Network C).

    - Switch 2 also connects to:

      - PC 2

      - PC 3

- DHCP Server:

  - Connected to Switch 7 (Network K).

- Web Server:

  - Connected to Switch 5 (Network H).

- NAT Router:

  - Router 16 in OSPF Area 2 and Router 8 in OSPF Area 1

## 2. Redistribution Configurations

### Router4 (RIP to OSPF Area 1):

### Router10 (OSPF Area 1 to EIGRP):

### Router18 (EIGRP to OSPF Area 2):

### 3. NAT Configurations

### Router8 (Network E):

### NAT Verification Commands:

### 4. ACL Configuration:

### 5. DHCP Configuration

### 6. Mail Server Configuration:

### Routing Table:

### 7. Network Verification Commands:

### EIGRP:

### OSPF:

### RIP:

### 8.Communications (ACL/ALL):

### ACL:

### Simple: